AFXXX and ACXXX (Antibodies identifiers from ABCD database, e.g. AG513) antibodies recognize (name of the target) by western blot

1Author, 2Author

1Affilitation 
 2 Affilitation

Abstract

The recombinant antibodies AFXXX and ACXXX (ABCD identifiers of the antibodies) detect by western blot (name of the target).

Introduction (3-15 lines)

Short description of the target. When available, please specify an identifier for the target (e.g: UniProt #P06213). Here, we describe the ability of (ABCD identifiers of the antibodies) to detect (name of the target) by western blot.

Materials & Methods

Antibodies: ABCD_AFXXX and ABCD_ACXXX, antibodies (ABCD nomenclature, http://web.expasy.org/abcd/) were produced by the Geneva Antibody Facility (http://unige.ch/medecine/antibodies/) and produced as minibodies/nanobodies with the antigen-binding scFv (or VHH in the case of nanobodies) portion fused to a rabbit IgG Fc. The synthesized scFv sequences (GeneArt, Invitrogen) correspond to the sequences of the variable regions joined by a peptide linker (GGGGS)3 . HEK293 suspension cells growing in HEK TF medium (Xell #861-0001, Sartorius), supplemented with 0.1% Pluronic F68 (Sigma #P1300), were transiently transfected with the vector coding for the VHH/scFv-Fc of each antibody. Supernatants (~50-80 mg/L) were collected after 4 days.

Antigen (3-5 lines): Description of the antigen used (cell lysate, transfected cell lysate / supernatant, purified protein,…)

Protocol: The recombinant human MOG-Fc-Avi-6xHis protein was purified from HEK293 cells (560 ng/µL) and diluted in reducing sample buffer (20.6% (w/v) sucrose, 100 mM Tris pH 6.8, 10 mM EDTA, 0.1% (w/v) bromophenol blue, 4% (w/v) SDS, 6% (v/v) -mercaptoethanol). Three different amounts of the protein were used in this study (225, 22.5 and 2.25 ng). 20 µL of each sample was migrated (200 V, 30 min) on a 4-15% acrylamide gel (Mini-PROTEAN® TGX™ Precast Gel, Biorad #456-1086), and transferred to a nitrocellulose membrane using a dry transfer system for 10 minutes (iBlot gel transfer device, Invitrogen #IB1001EU). The membranes were blocked during 1 hour in PBS containing 0.1% (v/v) Tween20 and 7% (w/v) milk, and washed three times for 15 minutes in PBS + 0.1% (v/v) Tween20. The membranes were incubated with AD946 (dilution 1:2 in PBS-Tween) overnight at 4°C, then washed three times for 15 minutes. The membranes were then incubated 1 hour with horseradish peroxidase-coupled goat anti-rabbit IgG (Biorad #REF, dilution 1:3000) and washed twice for 15 minutes and once for 5 minutes in PBS-Tween. The signal was revealed by enhanced chemiluminescence (ECL) (Amersham Biosciences) using a PXi-4 gel imaging systems (Syngene).

Results & Discussion

Antibodies AFXXX and ACXXX detect (name of the target )in sample (Fig. 1).

Insert figure (jpeg, png, 300 dpi ; 80 mm-wide, fonts and style: Arial 10 points)

Fig. 1. Specific binding of antibodies to name of the target, as detected by western blot. Short description of the figure.

References

Add any needed references in the APA format

Ex:

Rougeaux H, Kervarec N, Pichon R, Guezennec J. Structure of the exopolysaccharide of Vibrio diabolicus isolated from a deep-sea hydrothermal vent. Carbohydr Res. 1999 Nov 23;322(1-2):40-5. PMID: 10629947.

Conflict of interest

The authors declare no conflict of interest.

Data Availability Statement

The data that support the findings of this study are available from the corresponding author upon reasonable request.